US 6,038,389 A

Method of Modeling a Physical Process in a Material Environment (Krylov reduced-order model)

Institut Francais du Petrole (IFP)  -  Daniel Rahon, Mohamed Masmoudi  -  2000

Background & Problem Addressed

Finite-element modelling of an n-cell grid (e.g. reservoir fluid flow) is computationally expensive. A fast yet accurate simulation was needed for engineering use.

What the Patent Actually Describes

•  A model-order-reduction method - there is NO neural network in this patent.

•  Projection of the full n-cell system onto a Krylov subspace of dimension d much smaller than n (proper subspaces / base vectors).

•  Solution of the much smaller reduced problem, then reconstruction of the full-space solution.

•  Applied to fluid flow in an underground hydrocarbon reservoir.

Process Flow (Method Steps)

1.  Assemble the FEM modelling equations F(X)=0 over the n grid cells.

2.  Construct a Krylov subspace basis of dimension d << n.

3.  Project the system onto the basis to obtain a reduced problem.

4.  Solve the reduced problem cheaply.

5.  Reconstruct the full-space solution; iterate per time step.

Relevance to US 12,124,779 B2

This is exactly the reduced-basis prior art that US 12,124,779's FIG. 1 and background describe and improve upon (compress to a reduced basis, recurse). The patent argues its neural compression is more stable. Co-inventor Mohamed Masmoudi is also a named inventor of US 12,124,779 - a direct lineage from reduced-order modelling to the present method.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 6,038,389 A  -  verified against actual patent text