US 10,614,354 B2

Method and Apparatus for Implementing Layers on a Convolutional Neural Network Accelerator

Altera Corporation (Intel)  -  Utku Aydonat, Gordon R. Chiu, Andrew C. Ling  -  2020

Background & Problem Addressed

Real-time CNN inference needs efficient hardware; mapping convolutions to an FPGA must avoid off-chip memory bottlenecks.

What the Patent Actually Describes

•  A processing-element (PE) array compute engine for CNN layers on an FPGA.

•  Tiling of layers to fit on-chip BRAM; caching and double-buffering to hide DRAM latency.

•  Configurable to layer parameters; high throughput (the assignee's Arria-10 OpenCL DLA line).

Process Flow (Method Steps)

1.  Analyse the CNN and the target FPGA resources.

2.  Tile layers to fit BRAM; map tiles to the PE array.

3.  Stream activations / weights and accumulate.

4.  Double-buffer the next tile while computing the current one.

5.  Write back and proceed to the next tile / layer.

Relevance to US 12,124,779 B2

A deployment substrate for the patent's networks. Caveat: the patent's networks are tiny and sparse (194-291 links), so a dense systolic array is over-provisioned - the transferable idea is on-chip tiling and latency hiding.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 10,614,354 B2  -  verified against actual patent text