US 11,158,096 B1

Topology Optimization Using Straight-Through Estimators (photonic inverse design)

X Development LLC (Google X)  -  Martin Schubert  -  2021

Background & Problem Addressed

Inverse design of fabricable physical (photonic) devices: the material choice per region is discrete, which blocks gradients, and fabrication constraints must be respected.

What the Patent Actually Describes

•  A segmented design space where a 'paintbrush' assigns one of two materials to each segment to minimise a device loss.

•  Straight-through estimators (STE) to back-propagate gradients through the discrete material assignment.

•  Fabrication constraints (level-set) enforced during optimisation.

•  This is 'topology optimization' of a PHYSICAL DEVICE LAYOUT, not image reconstruction and not a neural-network graph.

Process Flow (Method Steps)

1.  Define the design as segments plus a loss.

2.  Compute touch-costs for each segment under each material pattern.

3.  Choose a segment + pattern by cost and assign material.

4.  Estimate the loss gradient via the straight-through estimator.

5.  Iterate until all segments are assigned and fabricable.

Relevance to US 12,124,779 B2

Shares the term 'topology optimization' but for a device layout, not an NN graph. The STE trick for differentiating through discrete choices could inform how candidate connections are relaxed/scored in the patent's additive phase.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 11,158,096 B1  -  verified against actual patent text