US 11,544,539 B2

Hardware Neural Network Conversion Method, Computing Device, Compiling Method and SW-HW Collaboration System

Tsinghua University  -  Youhui Zhang, Yu Ji  -  2023

Background & Problem Addressed

Deploying a neural-network application on (neuromorphic) hardware that has hardware constraints requires automatic conversion of the network.

What the Patent Actually Describes

•  Conversion of a neural-network connection graph into a 'hardware neural network' of basic-module virtual entities (ingress / egress nodes).

•  Support for multicast and reduction operations mapped onto module entities.

•  Compilation to satisfy a hardware constraint condition; weights calibrated to hardware (fixed-point) precision.

•  This is NN-to-hardware compilation, not sparse-matrix kernels.

Process Flow (Method Steps)

1.  Obtain the NN connection graph.

2.  Decompose it into hardware basic units.

3.  Map operations (matrix / reduction) onto module virtual entities.

4.  Assemble an equivalent hardware network.

5.  Calibrate / retrain weights to hardware precision; emit the hardware configuration.

Relevance to US 12,124,779 B2

A route to deploy the patent's network on constrained hardware (compile-to-hardware), useful for an inference-time benchmark. Not the CSR sparse-matmul the original summary claimed.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 11,544,539 B2  -  verified against actual patent text