US 11,616,471 B2

Systems and Methods for Connection Topology Optimization in Photovoltaic Arrays Using Neural Networks

Arizona Board of Regents (ASU)  -  Vivek S. Narayanaswamy et al.  -  2023

Background & Problem Addressed

Partial shading and mismatch reduce photovoltaic output; the electrical INTERCONNECTION of panels strongly affects losses.

What the Patent Actually Describes

•  A neural network that selects the array CONNECTION topology (which panels in series / parallel) to maximise net power.

•  Reported ~4-5% reduction in mismatch / power loss; a learned topology can outperform fixed series-parallel.

•  Switching-loss-aware reconfiguration.

Process Flow (Method Steps)

1.  Sense panel states / irradiance.

2.  NN predicts power for candidate connection topologies.

3.  Select the topology maximising net power above the switching cost.

4.  Reconfigure the array; repeat as conditions change.

Relevance to US 12,124,779 B2

Genuinely 'topology optimization' AND a physical-system NN application - a realistic physical-system data source / test case. It is array-wiring reconfiguration, not MPC / battery dispatch as the original summary claimed.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 11,616,471 B2  -  verified against actual patent text