US 2014/0067741 A1

Hybrid Interconnect Strategy for Large-Scale Neural Network Systems

International Business Machines (IBM)  -  Yasunao Katayama  -  2014

Background & Problem Addressed

Large neural-network hardware suffers signal-traffic bottlenecks between nodes.

What the Patent Actually Describes

•  A hybrid interconnect (fixed wired + adaptive / optical paths) for signal traffic in large NN hardware.

•  Identification of excess signal traffic and dynamic rerouting of part of it.

•  Optimisation of optical communication paths.

•  This is a hardware interconnect strategy, not architecture search.

Process Flow (Method Steps)

1.  Map the NN onto hardware nodes.

2.  Monitor signal traffic.

3.  Identify congested / excess traffic.

4.  Dynamically route part of it via adaptive (optical) paths; keep the rest on fixed wired paths.

Relevance to US 12,124,779 B2

Hardware interconnect / routing background for large-NN deployment. Not architecture determination.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 2014/0067741 A1  -  verified against actual patent text