US 2018/0322388 A1

Learning and Deployment of Adaptive Wireless Communications (autoencoder)

Virginia Tech Intellectual Properties  -  Timothy James O'Shea  -  2018

Background & Problem Addressed

Hand-engineered communication chains are sub-optimal on poorly-characterised channels.

What the Patent Actually Describes

•  An encoder ML network (transmitter) and a decoder ML network (receiver) trained end-to-end over an RF channel.

•  Optimisation of bit-error-rate / throughput / power; learned modulation.

•  Channel modelled as a differentiable middle layer during training.

Process Flow (Method Steps)

1.  Encoder maps a message to channel symbols under a power constraint.

2.  Channel model perturbs the signal.

3.  Decoder recovers the message.

4.  Jointly train encoder + decoder by end-to-end back-propagation.

5.  Deploy adaptively over the real channel.

Relevance to US 12,124,779 B2

Validates an end-to-end LEARNED encoder/decoder for a physical signal - conceptual support for the patent's jointly-trained compression block (FIG. 5 'learning on the total').

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 2018/0322388 A1  -  verified against actual patent text