US 2021/0357555 A1

Data-Driven Representation and Clustering-Discretization Method for Design Optimization / Performance Prediction of Material Systems

Northwestern University  -  Wing Kam Liu, Jiaying Gao, Cheng Yu, Orion L. Kafka  -  2021

Background & Problem Addressed

Multiscale material-system simulation is expensive; a fast reduced model was needed.

What the Patent Actually Describes

•  A data-driven representation of microstructure response fields at multiple scales.

•  Clustering of building blocks (microstructure volume elements) and computation of inter-cluster INTERACTION TENSORS.

•  Solving the governing PDE on the clustered (reduced) system for fast response prediction (self-consistent clustering analysis / mechanistic data science).

•  This is clustering reduced-order modelling for materials, NOT a generic CFD neural-network surrogate.

Process Flow (Method Steps)

1.  Sample MVE responses over material properties / boundary conditions.

2.  Cluster the building blocks.

3.  Compute inter-cluster interaction tensors.

4.  Solve the governing PDE on the clustered system.

5.  Predict the response; iterate in a multiscale scheme.

Relevance to US 12,124,779 B2

A reduced-order, physics-aware modelling method for physical systems (materials) - the same goal as the patent (fast surrogate of a physical system) but via clustering reduced-order modelling rather than a parsimonious neural network.

Reference document for US 12,124,779 B2 (Bompard et al., ADAGOS, Oct. 2024)  -  US 2021/0357555 A1  -  verified against actual patent text